1. Introduction

1.1 Project Overview

Agriculture is evolving rapidly with the adoption of Artificial Intelligence (AI), Internet of Things (IoT), Cloud Computing, and Data Analytics. However, a large percentage of farmers still rely on traditional irrigation methods that often lead to excessive water consumption, reduced crop productivity, and inefficient resource utilization.

DVR Smart Farming Solutions is an intelligent IoT-based agriculture platform designed to automate irrigation, monitor environmental conditions, provide AI-powered recommendations, and enable remote farm management through a cloud-based dashboard.

The system is designed not only for open-field farming but also for greenhouses, polyhouses, rooftop gardens, hydroponic farms, vertical farming systems, nurseries, and rare or high-value plants such as orchids and medicinal plants, where maintaining stable environmental conditions is critical.

The platform combines IoT sensors, cloud technologies, AI decision-making, weather forecasting, and an intuitive web dashboard into a single smart farming ecosystem.

1.2 Problem Statement

Agriculture is one of the most resource-intensive industries, and water scarcity has become one of the biggest challenges worldwide. Many farmers still depend on manual irrigation practices without accurate knowledge of soil moisture, temperature, humidity, or upcoming weather conditions.

These challenges become even more significant in environments such as greenhouses, polyhouses, orchid cultivation, and protected farming, where even slight environmental fluctuations can negatively affect crop quality and yield.

The major problems faced include:

- Excessive water wastage due to manual irrigation

- Over-irrigation and under-irrigation

- Lack of real-time monitoring

- Difficulty monitoring farms remotely

- No centralized monitoring platform

- High labour dependency

- Lack of AI-assisted decision making

- Unexpected weather affecting irrigation schedules

- Poor environmental control in greenhouses

- Crop damage caused by delayed responses

- High maintenance costs

- Inefficient resource utilization

Existing systems either provide only monitoring or only automation, without integrating AI, weather intelligence, cloud storage, and remote accessibility into a unified platform.

1.3 Proposed Solution

DVR Smart Farming Solutions addresses these challenges by integrating IoT devices, cloud computing, weather forecasting, and Artificial Intelligence into a single intelligent farming platform.

The system continuously monitors environmental conditions using ESP32-connected sensors. Sensor data is transmitted to Firebase Realtime Database over Wi-Fi, enabling real-time visualization through a modern web dashboard developed using Base44.

The dashboard provides live monitoring of:

- Soil Moisture

- Temperature

- Humidity

- Water Pump Status

- Irrigation Mode

- Weather Forecast

- AI Recommendations

Gemini AI analyses sensor values together with weather information and provides intelligent recommendations regarding irrigation, crop health, and environmental management.

The system automatically activates irrigation whenever soil moisture falls below a predefined threshold and stops irrigation once the desired moisture level is achieved.

Users can also manually control the irrigation system from anywhere through the dashboard.

2. Target Users

The proposed solution is designed for:

Farmers

- Vegetable Farmers

- Fruit Farmers

- Commercial Crop Farmers

- Organic Farmers

Protected Farming

- Greenhouses

- Polyhouses

- Shade Houses

- Hydroponic Farms

- Vertical Farming

High Value Plant Growers

- Orchid Farms

- Bonsai Gardens

- Medicinal Plant Farms

- Rare Plant Collectors

- Botanical Gardens

Urban Agriculture

- Rooftop Gardens

- Terrace Farming

- Smart Home Gardens

Research Organizations

- Agricultural Universities

- Plant Research Labs

- Government Agriculture Departments

3. Objectives

The objectives of DVR Smart Farming Solutions are:

- Automate irrigation based on soil moisture levels

- Minimize water wastage

- Improve crop productivity

- Enable real-time monitoring

- Provide AI-based farming recommendations

- Predict irrigation requirements using weather forecasts

- Reduce manual labour

- Support remote monitoring

- Improve sustainability

- Provide affordable precision farming

5.  Hardware Components

ESP32

Acts as the central IoT controller responsible for collecting sensor readings, controlling irrigation, connecting to Wi-Fi, and synchronizing data with Firebase.

Soil Moisture Sensor

Measures the water content present in soil.

Purpose:

- Detect dry soil

- Trigger irrigation

- Prevent overwatering

DHT11 Sensor

Measures:

- Temperature

- Humidity

Purpose:

- Crop monitoring

- Greenhouse monitoring

- Environmental analysis

Relay Module

Used for controlling the water pump electrically.

Functions:

- Pump ON

- Pump OFF

Water Pump

Provides irrigation automatically whenever soil moisture becomes low.

6. Software Components

Arduino IDE

Used to program the ESP32 firmware.

Base44

Used to rapidly develop the web dashboard with:

- Responsive UI

- Dashboard Cards

- AI Chat

- Weather Widget

- Charts

- Authentication

Firebase Realtime Database

Acts as the cloud database.

Stores:

- Soil Moisture

- Temperature

- Humidity

- Pump Status

- Automation Logs

- User Information

- AI Responses

Firebase Authentication

Provides secure login for users.

Gemini API

Used for AI decision making.

Gemini analyses:

- Soil Moisture

- Temperature

- Humidity

- Weather

- Crop Type

Outputs:

- Irrigation Advice

- Crop Suggestions

- Disease Prevention Tips

- Water Saving Recommendations

OpenWeather API

Provides:

- Current Temperature

- Humidity

- Rain Probability

- Wind Speed

- Weather Forecast

This information helps the AI engine determine whether irrigation should proceed or be postponed.

7. Detailed Communication Flow

Step 1 – Environmental Data Collection

The process begins when the sensors continuously monitor environmental conditions around the crops.

Soil Moisture Sensor

Measures:

- Soil moisture percentage

- Dryness level

- Water requirement

DHT11 Sensor

Measures:

- Temperature

- Relative Humidity

These sensors generate analog and digital signals that are transmitted directly to the ESP32 microcontroller.

Step 2 – Data Processing by ESP32

The ESP32 acts as the edge computing device of the entire system.

Its responsibilities include:

- Reading sensor values

- Filtering invalid readings

- Converting analog values into percentages

- Comparing moisture against the predefined threshold

- Deciding whether irrigation is required

- Connecting to Wi-Fi

- Sending processed data to Firebase

Step 3 – Wi-Fi Communication

After processing the sensor data, the ESP32 establishes a secure Wi-Fi connection.

Communication Protocol:

- IEEE 802.11 Wi-Fi

Purpose:

- Internet connectivity

- Cloud synchronization

- Remote monitoring

If the internet connection is unavailable:

- Sensor readings continue locally.

- Automatic irrigation still works using local thresholds.

- Data synchronization resumes when connectivity is restored.

This ensures the irrigation system remains functional even during temporary network failures.

Step 4 – Firebase Cloud Communication

The ESP32 uploads all sensor readings to Firebase Realtime Database.

Data transmitted includes:

- Temperature

- Humidity

- Soil Moisture

- Pump Status

- Irrigation Mode

- Timestamp

Step 5 – Dashboard Communication

The dashboard developed using Base44 establishes a real-time connection with Firebase.

Whenever Firebase detects new data:

- Dashboard updates automatically

- Graphs refresh instantly

- Pump status changes immediately

- Moisture gauge updates

- Temperature cards refresh

- Humidity cards refresh

No manual page refresh is required.

Step 6 – Weather API Communication

The dashboard requests weather information from the OpenWeather API.

Retrieved information includes:

- Current temperature

- Humidity

- Rain forecast

- Wind speed

- Cloud coverage

- Weather conditions

Step 7 – AI Decision Making using Gemini

Gemini AI receives information from multiple sources.

Input Data

From Firebase:

- Soil Moisture

- Temperature

- Humidity

- Pump Status

From Weather API:

- Rain Probability

- Weather Forecast

- Wind Speed

From User:

- Crop Type

- Farm Type

- Irrigation Mode

Gemini processes this information using natural language reasoning and predefined prompts.

Example Decision:

Soil moisture is low (18%), but rain is expected within 2 hours. Delay irrigation to conserve water.

Another example:

Temperature is 36°C with humidity at 22%. Increase irrigation frequency for tomato crops.

The AI-generated recommendation is stored in Firebase and displayed on the dashboard.

Step 8 – Automatic Irrigation Decision

The ESP32 continuously compares soil moisture against a predefined threshold.

Example:

Threshold = 30%

Current Moisture = 18%

18 < 30

↓

Pump ON

When sufficient moisture is reached:

Current Moisture = 42%

42 > 30

↓

Pump OFF

This logic prevents both overwatering and underwatering.

Step 9 – Manual Control

The farmer can override automatic operation through the dashboard.

Manual operations include:

- Start Pump

- Stop Pump

- Switch between Automatic and Manual Mode

Communication Flow:

Farmer

↓

Dashboard

↓

Firebase

↓

ESP32

↓

Relay

↓

Water Pump

The ESP32 continuously listens for updates in Firebase and executes commands in real time.

Step 10 – Alerts and Notifications

Whenever abnormal conditions occur, the system generates alerts.

Examples:

- Low soil moisture

- High temperature

- Excess humidity

- Pump failure (if implemented)

- Internet disconnection (if implemented)

Alerts are displayed on the dashboard and can be extended in future to mobile notifications.

Step 11 – Data Logging

Every event is recorded in Firebase for future analysis.

Stored events include:

- Sensor readings

- Pump ON/OFF history

- Irrigation duration

- Weather data

- AI recommendations

- User actions

- System timestamps

These records help analyze water usage, crop conditions, and system performance over time.

Complete Communication Workflow

1. Sensors collect environmental data

│

▼

2. ESP32 reads and validates sensor values

│

▼

3. ESP32 processes moisture thresholds

│

▼

4. ESP32 uploads data to Firebase via Wi-Fi

│

▼

5. Firebase synchronizes data with Dashboard

│

▼

6. Dashboard fetches weather from OpenWeather API

│

▼

7. Gemini AI analyzes:

• Sensor Data

• Weather Data

• Crop Type

│

▼

8. AI generates recommendations

│

▼

9. Dashboard displays:

• Live Sensor Data

• Weather Updates

• AI Advice

• Pump Status

│

▼

10. ESP32 controls irrigation automatically

│

▼

11. Farmer can manually override pump operation

│

▼

12. All events are logged in Firebase

8. Firebase Data base Jason structure

{

"farm": {

"nodes": {

"node_01": {

"device_status": "OFFLINE",

"humidity": "Data not received",

"id": "node_01",

"last_updated": 1783187542191,

"moisture": "Data not received",

"name": "Zone 1 - North Field",

"pump": "OFF",

"pump_reason": "Data not received",

"source": "hardware",

"temperature": "Data not received"

},

"node_02": {

"device_status": "OFFLINE",

"humidity": "Data not received",

"id": "node_02",

"last_updated": 0,

"moisture": "Data not received",

"name": "Zone 2 - South Field",

"pump": "OFF",

"pump_reason": "Data not received",

"source": "hardware",

"temperature": "Data not received"

},

"node_03": {

"device_status": "OFFLINE",

"humidity": "Data not received",

"id": "node_03",

"last_updated": 0,

"moisture": "Data not received",

"name": "Zone 3 - East Field",

"pump": "OFF",

"pump_reason": "Data not received",

"source": "hardware",

"temperature": "Data not received"

},

"node_04": {

"device_status": "OFFLINE",

"humidity": "Data not received",

"id": "node_04",

"last_updated": 0,

"moisture": "Data not received",

"name": "Zone 4 - West Field",

"pump": "OFF",

"pump_reason": "Data not received",

"source": "hardware",

"temperature": "Data not received"

},

"node_05": {

"device_status": "OFFLINE",

"humidity": "Data not received",

"id": "node_05",

"last_updated": 0,

"moisture": "Data not received",

"name": "Zone 5 - Center Field",

"pump": "OFF",

"pump_reason": "Data not received",

"source": "hardware",

"temperature": "Data not received"

}

}

}

}

9. Complete Working Model

- Sensors continuously monitor the farm environment.

- ESP32 reads all sensor values.

- ESP32 uploads the readings to Firebase over Wi-Fi.

- Firebase synchronizes the data with the dashboard in real time.

- The dashboard displays live sensor readings and pump status.

- OpenWeather API retrieves weather forecasts.

- Gemini AI combines sensor data, weather information, and crop type to generate intelligent recommendations.

- Farmers receive actionable insights through the dashboard.

- If automatic mode is enabled, the ESP32 switches the water pump on or off according to soil moisture thresholds.

- Every sensor update, AI recommendation, and pump event is logged for future analysis.